Additional Information

Arystox Ceramic Abrasive Flexi-Pads (double-sided) are a high-performance sanding solution made with advanced abrasive technology for faster preparation, longer life and significant fatigue reduction.

🟣 Self-Sharpening Ceramic Oxide Grain

Microcrystalline grain structure fractures during use to continuously expose fresh cutting edges, keeping the abrasive sharp for longer

Delivers a faster cut with less effort compared to traditional aluminium oxide

🟣 Flexible Foam Backing

Adapts to contours while maintaining even contact with the surface, giving better control and reducing the risk of over-sanding

Particularly effective on shaped areas such as skirting boards, baseboards, picture rails, spindles, handrails, panelled doors, pipes and radiators

Also suitable for flat surfaces

🟣 Non-Stick Stearate Coating

Resists particle build-up and clogging, allowing the abrasive to keep cutting efficiently for longer

Reduces heat build-up, helping prevent material from sticking, especially when sanding fillers and painted surfaces

The anti-clog coating removes the need for gaps between grains (commonly found in standard open-coat abrasives), allowing full grain coverage (closed-coat) for more cutting points and faster sanding

🟣 Electrostatically-Aligned Grain

Unlike conventional abrasives where grains are randomly positioned, the grains are aligned so their sharpest edges face outward

This means more cutting points are working from the start, delivering a faster, more efficient cut with less pressure

Produces a more consistent scratch pattern for a smoother, more predictable finish

🟣 Works Wet or Dry

Suitable for both dry sanding and wet sanding applications

When used wet, helps reduce airborne dust, control heat and produce a finer, more refined finish

Wash after use to unclog and extend lifespan

Product Care

Arystox Flexi-Pads Usage Guide

🟣 Choose the Right Grit

P60 – Extra-Coarse

Rapid removal of old coatings; shaping bare wood; levelling deep gouges, scratches and dents

P80 – Coarse

Removing old coatings; correcting significant imperfections; levelling over-filled areas; rust removal

P120 – Medium

Removing imperfections; keying surfaces prior to coating; preparing bare wood; smoothing plaster and filler

P150 – Medium-Fine

Keying surfaces prior to coating; removing scratches from coarser abrasives; final sanding of filler; preparing bare wood

P180 – Fine

Final preparation of bare wood; removal of brush and roller marks; reducing visibility of scratches from coarser abrasives

🟣 Before Use

🔍 Inspect the pad for dust or debris from previous use to ensure optimal sanding performance

✔ Choose the correct grade for the task, working from coarse to fine for a superior finish

⚠️ Test on a small area before full application to ensure the desired sanding effect

🟣 Sand with Consistent Motion

Use even, controlled strokes to maintain a uniform finish

For flat surfaces, keep full pad contact

For profiles, allow the pad to naturally flex around the shape

🟪 Note on Sanding Dust

Ceramic abrasives may leave a fine purple dust on the surface during use

These are dry particles of abrasive grain that sit within the surface texture and do not stain

Always wipe this grain dust off before applying coatings

For best results, use an Tack Cloth

💜 Maximise Performance

Whilst the stearate coating reduces clogging during dry sanding, tapping the pad periodically to remove dust build-up will maintain peak performance

For a full clean after use, rinse the pad with water to remove embedded dust, then leave to dry for maximum cutting power on the next job

Rotate the pad while sanding to distribute wear evenly and extend its lifespan

When wet sanding, rinse regularly to unclog

Replace the pad once cutting performance noticeably reduces

🟣 Pro Tips

Use lighter pressure than with aluminium oxide abrasives

Always work through grits progressively (e.g. P80 → P120 → P150 → P180)

Storage

Store in a dry, well-ventilated area to maintain abrasive quality

Keep away from direct sunlight and extreme heat, which can affect the foam backing